# PONDICHERRY UNIVERSITY

# (A Central university)

SCHOOL OF ENGINEERING AND TECHNOLOGY DEPARTMENT OF COMPUTER SCIENCE

M.Sc. Computer Science

## NAME	:	KHAIF HASAN

REG. NO.	:	20384114

## SEMESTER	:	9TH Semester

SUBJECT	:	CSSC 513 - WEB TECHNOLOGY AND COMPUTER NETWORKS LAB

PONDICHERRY UNIVERSITY

(A Central university)

SCHOOL OF ENGINEERING AND TECHNOLOGY DEPARTMENT OF COMPUTER SCIENCE

M.Sc. Computer Science PRACTICAL LAB RECORD BONAFIDE CERTIFICATE

## This is to certify that this is a Bonafide record of practical work done by KHAIF HASAN, having  Reg. No. 20384114 semester - IX from the month July 2024 to December 2024.

### FACULTY IN-CHARGE

SUBMITTED FOR THE PRACTICAL EXAM HELD ON:

### INTERNAL EXAMINER	EXTERNAL EXAMINER

AIM:

# NETWORKTOPOLOGY

To configure a basic Network Topology using Cisco Packet Tracer.

### PROCEDURE:

- Place the devices:

  - From the Devices menu, select Routers and choose the 1841 Router. Place it in the center.

  - Go to Switches, select two 2960 switches, and place them on the left and right of the router.

  - From End Devices, select PC and place three PCs connected to the left switch and three PCs connected to the right switch.

### Configure Connections:

  - Router to Switches:

    - Use the Copper Straight-Through Cable tool to connect:

      - Router FastEthernet0/0 to Switch0 FastEthernet0/1 (left switch).

      - Router FastEthernet0/1 to Switch1 FastEthernet0/1 (right switch).

### Switches to PCs:

    - For Switch0, connect each of its FastEthernet ports (e.g., FastEthernet0/2, FastEthernet0/3, and FastEthernet0/4) to PC0, PC1, and PC2.

### For Switch1, connect its FastEthernet ports (e.g., FastEthernet0/2, FastEthernet0/3, and FastEthernet0/4) to PC3, PC4, and PC5.

- Configure IP Addresses:

### Router Interfaces:

    - Click on Router0, go to Config > FastEthernet0/0, and set:

      - IP Address: 192.168.10.1

      - Subnet Mask: 255.255.255.0

      - Turn on the interface by clicking on Port Status.

    - Go to FastEthernet0/1 and set:

      - IP Address: 192.168.11.1

      - Subnet Mask: 255.255.255.0

      - Turn on the interface by clicking on Port Status.

### PCs:

    - For PC0, PC1, and PC2 (connected to the left switch):

      - Set IP addresses within the 192.168.10.0 network (e.g., 192.168.10.2, 192.168.10.3, 192.168.10.4) with a Subnet Mask of 255.255.255.0.

      - Set the Default Gateway to 192.168.10.1.

    - For PC3, PC4, and PC5 (connected to the right switch):

      - Set IP addresses within the 192.168.11.0 network (e.g., 192.168.11.2, 192.168.11.3, 192.168.11.4) with a Subnet Mask of 255.255.255.0.

      - Set the Default Gateway to 192.168.11.1.

### Test Connectivity:

  - Use the Ping Tool from PC0 to PC3 (or any PC on a different subnet) to verify connectivity.

### DIAGRAM :

RESULT:

Thus the configuration of the basic Network Topology is successfully pings indicate proper           configuration and connectivity between the subnets.

AIM:

## VLAN

To configure VLANs on a switch and router in Cisco Packet Tracer to enable

communication between devices in different VLANs through inter-VLAN routing.

### PROCEDURE:

Step 1: Set Up the Devices

- Place the Router, Switch, and PCs in the workspace.

- Connect the Router to the Switch using a cross-over cable from Router FastEthernet0/0 to Switch FastEthernet0/20.

- Connect each PC to the Switch:

  - PC1 and PC2 will be in VLAN 10 (connect to any ports like Fa0/1 and Fa0/2).

  - PC3 and PC4 will be in VLAN 20 (connect to any ports like Fa0/3 and Fa0/4).

### Step 2: Configure VLANs on the Switch

- Access the Switch CLI:

  - Click on the switch, go to the CLI tab.

### Enter Configuration Mode:

config# enable

config# configure terminal

### Create VLANs:

  - VLAN 10:

config# vlan 10 config# name VLAN10 config# exit

### VLAN 20:

config# vlan 20 config# name VLAN20 config# exit

### Assign Ports to VLANs:

  - For VLAN 10 (PC1 and PC2): config# interface FastEthernet0/1 config# switchport mode access config# switchport access vlan 10 config# exit

config# interface FastEthernet0/2 config# switchport mode access config# switchport access vlan 10 config# exit

### For VLAN 20 (PC3 and PC4):

config# interface FastEthernet0/3 config# switchport mode access config# switchport access vlan 20 config# exit

config# interface FastEthernet0/4 config# switchport mode access config# switchport access vlan 20 config# exit

### Configure the Trunk Port:

  - Set the port connected to the router as a trunk port (e.g., FastEthernet0/20).

config# interface FastEthernet0/20 config# switchport mode trunk config# exit

### Step 3: Configure the Router for Inter-VLAN Routing

- Access the Router CLI:

  - Click on the router, go to the CLI tab.

### Enter Configuration Mode:

config# enable

config# configure terminal

### Configure Subinterfaces for Each VLAN:

  - Subinterface for VLAN 10:

config# interface FastEthernet0/0.10 config# encapsulation dot1Q 10

config# ip address 192.168.1.100 255.255.255.0 config# exit

### Subinterface for VLAN 20:

config# interface FastEthernet0/0.20 config# encapsulation dot1Q 20

config# ip address 192.168.2.100 255.255.255.0 config# exit

### Enable the Main Interface:

  - Make sure the main interface FastEthernet0/0 is up. config# interface FastEthernet0/0

config# no shutdown config# exit

### Step 4: Configure IP Addresses on PCs

- For PC1 (VLAN 10):

o IP Address: 192.168.1.1

o Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.100

- For PC2 (VLAN 10):

o IP Address: 192.168.1.2

o Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.100

- For PC3 (VLAN 20):

o IP Address: 192.168.2.1

o Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.2.100

- For PC4 (VLAN 20):

o IP Address: 192.168.2.2

o Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.2.100

### Step 5: Test Connectivity

- Ping from PC1 to PC2 within VLAN 10 (should succeed).

- Ping from PC3 to PC4 within VLAN 20 (should succeed).

- Ping between PCs in different VLANs (e.g., PC1 to PC3) to verify inter-VLAN routing (should also succeed).

DIAGRAM:

RESULT:

Thus the configuration of VLAN is successfully done and the ping from one PC to other PCs is verified.

AIM:

FIREWALL

To setup an network based firewall using Cisco Packet Tracer.

### PROCEDURE:

Step 1: Replace Router_A with Firewall_1

- a. Remove Router_A and replace it with Firewall_1.

- b. Connect the FastEthernet 0/0 interface on Firewall_1 to the FastEthernet 0/1 interface on Switch_A.

Connect the FastEthernet 0/1 interface on Firewall_1 to the Ethernet 6

interface of the ISP cloud.

(Use straight-through cables for both connections.)

- c. Confirm that the host name of Firewall_1 is Firewall_1.

- d. On Firewall_1, configure the WAN IP address and subnet mask for the

FastEthernet 0/1 interface as 209.165.200.225 and 255.255.255.224.

- e. Configure the LAN IP address and subnet mask for the FastEthernet 0/0

interface on Firewall_1 as 192.168.1.1 and 255.255.255.0.

### Step 2: Verify the Firewall_1 Configuration

- a. Use the show run command to verify your configuration. This is a partial example of the output:

Firewall_1#show run Building configuration... hostname Firewall_1

!

interface FastEthernet0/0

ip address 192.168.1.1 255.255.255.0

ip nat inside duplex auto speed auto

!

interface FastEthernet0/1

ip address 209.165.200.225 255.255.255.224

ip access-group 100 in ip nat outside

duplex auto speed auto

!

interface Vlan1 no ip address shutdown

!

ip nat inside source list 1 interface FastEthernet0/0 overload ip classless

ip route 192.168.2.0 255.255.255.0 192.168.1.2

ip route 192.168.3.0 255.255.255.0 192.168.1.3

!

access-list 1 permit 192.168.0.0 0.0.255.255 access-list 100 deny ip any host 209.165.200.225

<output omitted>

!

end

- b. From PC_B, ping 209.165.200.225 to verify that the internal computer can access the Internet.

PC>ping 209.165.200.225

Pinging 209.165.200.225 with 32 bytes of data:

Reply from 209.165.200.225: bytes=32 time=107ms TTL=120 Reply from 209.165.200.225: bytes=32 time=98ms TTL=120 Reply from 209.165.200.225: bytes=32 time=104ms TTL=120 Reply from 209.165.200.225: bytes=32 time=95ms TTL=120 Ping statistics for 209.165.200.225:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss), Approximate round trip times in milli-seconds: Minimum = 95ms, Maximum = 107ms, Average = 101ms

- c. From privileged EXEC mode on Firewall_1, save the running configuration

to the startup configuration using the copy run start command.

### DIAGRAM:

RESULT:

Hence, the firewall setup is configured within the network interface.

AIM:

ROUTER CONFIGURATION

Basic configuration of Router using Cisco Packet Tracer.

### PROCEDURE:

- Place the Router:

  - From the Devices menu, go to Routers and select a router model (e.g., 1841 Router). Place it on the workspace.

### Add Network Devices (Optional):

  - Add switches and PCs if you want to connect multiple devices to the router, creating different networks or subnets.

### Connect Devices:

  - Use Copper Straight-Through Cable to connect the router to other devices.

  - Connect Router’s FastEthernet or GigabitEthernet ports to the switches or directly to PCs, depending on the setup.

### Enter Router Configuration Mode:

  - Click on the router, then go to the CLI (Command Line Interface) tab.

  - When prompted, type no if it asks if you want to enter the initial configuration dialog.

### Access the Router’s Global Configuration Mode:

  - Type enable to enter privileged EXEC mode.

  - Type configure terminal to enter global configuration mode.

### Configure Router Interfaces:

  - Enter interface configuration mode for each interface you want to configure:

    - For FastEthernet0/0:

config# interface FastEthernet0/0

    - Set the IP address and subnet mask:

config# ip address 192.168.10.1 255.255.255.0

    - Turn on the interface:

config# no shutdown

    - Exit the interface configuration: config# exit

  - Repeat the process for FastEthernet0/1 (or any other interface): config# interface FastEthernet0/1

config# ip address 192.168.11.1 255.255.255.0 config# no shutdown

config# exit

### Configure Routing (Optional, if using multiple networks):

  - For static routing, type:

config# ip route 192.168.11.0 255.255.255.0 192.168.10.2

  - This step is optional if you only need basic routing between directly connected networks.

### Save the Configuration:

  - To save the configuration, exit global configuration mode by typing exit until you return to the privileged EXEC mode.

  - Type:

config# write memory

  - Alternatively, use copy running-config startup-config to save the configuration to non-volatile memory.

### Test Connectivity:

  - Connect PCs to the router via switches or directly.

  - Assign IP addresses and default gateways to each PC in their respective network.

  - Use the Ping Tool to test communication between devices in different networks.

### DIAGRAM:

RESULT:

The basic router configuration is made using PCs, Switches and Router which connects all seamlessly.